(SAMPLE)

FLIGHT TESTING RECORD

In chronological sequence, describe all qualifying flight test experience for which you have acted as PILOT of the test vehicle.  See the instructions for definitions of qualifying experience.  Be as explicit as possible.  Show dates with a breakdown of flights by month and year.  Use supplementary sheets as necessary.

| Month/Year | Number of Flights | Crew Position | Aircraft | DESCRIPTION OF TESTING | FOR OFFICE USE ONLY |
| --- | --- | --- | --- | --- | --- |
| APR 75 | 3 | Proj Pilot | A-7D | A-7D DIGITAC. AF Flight Dynamics Lab research and development program testing a multimode digital flight control system implemented in an A-7D replacing the production analog system; prototype hardware and software. Included initial control law development, gain level and envelope expansion, classic stability and control investigation, and task-oriented handling qualities evaluation. |  |
| MAY 75 | 4 | Proj Pilot | A-7D | A-7D DIGITAC. Same as above. |  |
| JUN 75 | 4 | Proj Pilot | A-7D | A-7D DIGITAC. Same as above. |  |
| JUL 75 | 5 | Proj Pilot | A-7D T-38 | Same as above. Additional: T-38 LEAD-IN-FIGHTER. Initial development of a prototype T-38B aircraft modified to provide a weapons delivery capability. Included initial flights with centerline-mounted training weapons (SUU-ll/SUU-20) to determine performance, flying qualities, structural integrity, stores compatibility, separation, gas ingestion, ballistics verification, and accuracy. Tests also included increased gross weight takeoff and single engine performance. |  |
| AUG 75 | 7 | Proj Pilot | A-7D T-38 | A-7D DIGITAC/T-38 LEAD-IN-FIGHTER. Same as above. |  |
| MAR 82 | 7 | Proj Pilot | A-10B | A-10B HIGH ANGLE OF ATTACK. Initial experimental tests to determine stall/post-stall/ departure and recovery characteristics of the A-10 aircraft modified to two-seat configuration. |  |
| JAN 83 | 4 | Proj Pilot | F-15C | F-15C HIGH ANGLE OF ATTACK. Initial experimental tests of F-15C modified with conformal fuel tanks installed to determine stall/ departure/ spin susceptibility/recovery characteristics at significantly higher angles of attack than standard F-15. Included one unintentional six-turn spin and successful recovery. |  |
| FEB 83 | 8 | Proj Pilot | F-15C | F-15C. Same as above. Additional: F-15 DUAL ROLE FIGHTER. Initial basic performance and flying qualities of F-15C with enhanced air-to-ground store loadings not previously flown. Also testing with conformal fuel tanks installed at increased gross weights to 75,000#; takeoff, cruise/ maneuvering performance, and basic flying qualities. |  |